# Laboratory analyses on split core to assess the geothermal potential of the South Slave Region, Northwest Territories

Report for SR-2023-002 and SR-2023-004

Submitted: 2025

Michaël Thibault and Jasmin Raymond

## Introduction

The Energy Geosciences Group of the Northwest Territories Geological Survey (NTGS) is currently partnering with the Institut national de la recherche scientifique (INRS) to assess the geothermal potential of the South Slave Region (Northwest Territories, Canada). This project, initiated in summer 2022, aims to evaluate subsurface thermal properties and temperature to evaluate the performances of closed-loop geothermal systems for communities of Hay River, Fort Providence, Kakisa and Enterprise.

The South Slave Region in known for the potentially highest heat flow (>100 mW/m2) in the Western Canadian Sedimentary Basin (Rajaobelison et al., 2024; Thibault et al., 2024). Yet, no in-depth study has been conducted on the communities’ deep geothermal resources.

This report objective is to improve our knowledge of the rock thermal properties in the South Slave region, building upon our prior research (Rajaobelison et al., In press).

## Summary of the Sampling and Laboratory Analysis Procedures

Core sampling was authorized and approved by the Office of the Regulator of Oil and Gas Operations (OROGO) in July of 2023 for SR-2023-002 and sampling at the Geological Survey of Canada’s Core Repository in Calgary was conducted August 2023 with a total of 22 samples from 7 wells (Table 1). For SR-2023-004, sampling was authorized in November 2023 and sampling was made in December 2025, with 8 samples from 4 wells (Table 2).

Thermal properties (thermal conductivity, thermal diffusivity and heat capacity) of rock samples were measured at the Laboratoire ouvert de géothermie (LOG) at INRS with an optical scanning instrument developed by Popov et al. (2023).

The parameters automatically computed from analysis with the optical scanning device are, λ the thermal conductivity [W m-1 K-1], α the thermal diffusivity [m-2 s-1], G the coefficient of variation (%), and IF the inhomogeneity factor. Rajaobelison et al. (In press) can be consulted for an exhaustive description of the laboratory analysis.

Table 1. Core samples collected from wells for SR-2023-002 request.

| Well | Depth (m) | Formation | Lithology |
| --- | --- | --- | --- |
| A-66 | 538.7 | Mirage Point | Red Beds |
| A-66 | 547.82 | Mirage Point | Red Beds |
| A-66 | 549.43 | La Loche | Sandstone |
| A-66 | 552.42 | La Loche | Sandstone |
| A-70 | 642.63 | Keg River | Limestone |
| A-70 | 646.85 | Keg River | Limestone |
| A-70 | 689.82 | Chinchaga | Anhydrite |
| A-70 | 690.65 | Chinchaga | Anhydrite |
| C-19 | 393.5 | Hay River | Shale |
| C-19 | 395.63 | Hay River | Shale |
| I-57 | 508.6 | Chinchaga | Anhydrite |
| I-57 | 509.3 | Chinchaga | Anhydrite |
| B-75 | 443.63 | Horn River | Limestone |
| B-75 | 446.83 | Horn River | Limestone |
| I-08 | 416.51 | Muskwa | Limestone |
| I-08 | 424.17 | Muskwa | Shale |
| O-25 | 874.14 | Chinchaga | Dolostone |
| O-25 | 879.55 | Chinchaga | Anhydrite |
| O-25 | 881.7 | La Loche | Unknown |
| O-25 | 882.19 | La Loche | Unknown |
| O-25 | 885.93 | Precambrian basement | Granitoid |
| O-25 | 886.51 | Precambrian basement | Granitoid |

Table 2. Core samples collected from wells for SR-2023-004 request.

| Well | Depth (m) | Formation | Lithology |
| --- | --- | --- | --- |
| A-21 | 418 | Precambrian basement | Granitoid |
| A-21 | 418,8 | Precambrian basement | Granitoid |
| G-68 | 388,6 | Precambrian basement | Granitoid |
| G-68 | 389 | Precambrian basement | Granitoid |
| H-27 | 520,75 | Precambrian basement | Granitoid |
| H-27 | 520,8 | Precambrian basement | Granitoid |
| M-02 | 432,8 | Precambrian basement | Granitoid |
| M-02 | 433,8 | Precambrian basement | Granitoid |

## Summary and Interpreted Results

Two articles and a thesis were published including results from the analysis performed. A scientific article was published by Rajaobelison et al. (2024) in August 2024 and a conference paper was published by Thibault et al. (2024) in October 2024. An additional MSc thesis by Thibault (2025) was published in June 2025. These are included in the Appendix (1-3). The first paper focuses on a thermostratigraphic and heat flow assessment of the four South Slave communities, describing the development of 1D temperature models based on vintage oil and gas wells data to evaluate subsurface temperature within the sedimentary sequence below these communities (Rajaobelison et al., 2024). The second paper, presented at a Geothermal Rising Conference, assesses the temperature distribution in the Precambrian basement, underlying the sedimentary basin and providing insights into deep geothermal conditions below the communities (Thibault et al., 2024). The thesis covers these aspects in greater detail, providing more comprehensive interpretations of the results.

Appendix 4 present raw data associated to the thermal properties analysis from each sample.

Ongoing work includes numerical simulations of closed-loop geothermal systems, which requires as input subsurface thermal properties. Future work will focus on developing an open-access geothermal database that will include detailed information on the thermal properties of rock samples.

## Acknowledgement

We would like to thank OROGO and CGC CRC for providing access to the cores and assistance with sampling. Taís Fontes Pinto helped for sampling the cores in December 2023. The Northern Scientific Training Program (NSTP) and NTGS provided funding for this project.

## Reference

Popov, Y., Lippmann, E., & Rauen, A. (2023). TCS-Manual. Thermal conductivity (TC) and thermal diffusivity (TD) Scanner. Version 15.11.2023. https://www.schwartech.de/index_en.html#/tcs_en

Rajaobelison, M., Thibault, M., Comeau, F.-A., Raymond, J., Smejkal, E. J., & Terlaky, V. (2024). Thermostratigraphic and Heat Flow Assessment of the South Slave Region in the Northwest Territories, Canada. Energies, 17(16), Article 16. https://doi.org/10.3390/en17164165

Rajaobelison, M., Thibault, M., Comeau, F.-A., Raymond, J., & Terlaky, V. (In press). Laboratory analyses on split core samples from wells to assess the geothermal potential of the South Slave Region, Northwest Territories, Canada (Open Report Nos. 2023–015; p. 71). Institut national de la recherche scientifique & Northwest Territories Geological Survey.

Thibault, M. (2025). Evaluation of the deep geothermal potential below four South Slave communities, Northwest Territories, Canada [Thesis, Reykjavik University, Iceland School of Energy]. https://skemman.is/handle/1946/50759

Thibault, M., Comeau, F.-A., Raymond, J., Rajaobelison, M., Terlaky, V., & Newson, J. (2024). Evaluation of the Precambrian Basement Temperature Below Four South Slave Communities, Northwest Territories, Canada. GRC Transactions, 48, 28.

## Appendix

- Rajaobelison et al., (2024) paper

- Thibault et al. (2024) paper

- Thibault (2025) thesis

- Thermal properties raw data